# Production Logistics & Capacity Analysis

Project: Shelly's Indigenous Bistro — 1M Meals/Day RTE Facility
Date: August 11, 2026

# 1. The Chilling Bottleneck Calculation

To reach 1,000,000 meals per day at an average weight of 450g, the plant must process 450,000 kg per day.

Over a 17-hour effective production window (20h shift - 15% OEE/Sanitation), the required instantaneous rate is 26,500 kg/hr.

## Current Hot Food Limit (Chilling Constraint):

Blast Chiller Fleet: 10 Units.

Load per Chiller: 10 Trolleys (200L each).

Total Capacity per Cycle: 100 Bins x 200 kg = 20,000 kg.

Cooling Cycle Time: 120 Minutes (2 hours).

Hourly Chilling Throughput: 10,000 kg/hr.

Hot Food Possible: 170,000 kg/day (approx. 377,000 meals).

# 2. Equipment Specs & Canadian Landed Costs

To provide full assurance for management, the following equipment has been sized and priced for the Canadian market (including SPE-1000 field evaluation fees).

## 2.1 200L Euro-Bin (Meat Trolley)

Dimensions: 790mm (L) x 675mm (W) x 700mm (H)

Material: SS304 Mirror Finish (DIN 9797 standard)

Quantity: 400 Units

Landed Cost (CAD): ~$500/unit | Total: $200,000 CAD

## 2.2 Industrial 10-Trolley Blast Chiller

External Size: 5.2m (L) x 1.8m (W) x 2.4m (H) per unit

Power: 30HP - 50HP per unit (Electrical service must support ~400kW total)

Quantity: 10 Units

Landed Cost (CAD): ~$17,000/unit (Includes SPE-1000 cert) | Total: $170,000 CAD

# 3. Floor Space & Facility Impact

The transition to 200L batch chilling requires dedicated space for bin circulation and walk-in chiller cabinets.

Chilling Area: ~250 m² (2,700 sq.ft)

Bin Storage/Wash: ~150 m² (1,600 sq.ft)

Total Logistics Footprint: ~400 m² (4,300 sq.ft)

# 4. Operational Strategy: Parallel Processing

We will run the facility with two distinct logistics flows:

## Stream A: The "Heat" Flow (38% of Capacity)

Cooking: Kettles and ovens run on 45-60 min staggered cycles.

Transfer: Decanted via mobile lobe pumps into 200L Euro-bins.

Chilling: Bins wheeled into the 10 blast chillers for a 2-hour drop to 41°F.

Buffer: Held in WIP cold store before being called to packaging lines 1-4.

## Stream B: The "Cold" Flow (62% of Capacity)

Prep: Vegetables and fruits washed and cut via high-capacity continuous lines (Sormac/Urschel).

Assembly: Sandwich and wrap lines run as high-velocity assembly belts.

Packaging: Items fed directly to MAP sealers and cup fillers on lines 5-12.

Assurance: Because these items are never heated, they do not impact the blast chilling bottleneck, allowing us to "overflow" the 1M meal target by increasing prep speed.

# 5. Scaling the Plan

If Management requires more than 38% hot food, we must add blast chillers at a ratio of:

+1 Blast Chiller (10-trolley) = +1,000 kg/hr Hot Food Capacity.

Cost impact: ~$6,500 per additional unit + floor space.

Recommendation: Proceed with a 40/60 Hot/Cold split for the initial Wave 1 procurement and facility layout.